# AGRICULTURAL DRIP IRRIGATION FIELD PLUMBING & EXECUTION MANUAL

## 2-Acre Commercial Moringa Farm (PKM-1 & Bhagya Cultivars)

### DOCUMENT CONTROL & METADATA

- Project: 2-Acre High-Density & Commercial Moringa Irrigation System

- Location: Agricultural Farm Sector

- Engineering Standard: IS 12786 (Irrigation Laterals) / IS 4985 (PVC Pressure Pipes)

- Authoring Suite: ArtCraft Engineering Toolchain (CADCraft, VectorCraft, GridCraft, WordCraft)

- Document Version: Revision 1.0 (Field Approved)

- Scope: Head Unit, Mainline Network, Overhead Double-Tee Risers, Inline & Online Laterals, Flushing & Commissioning

## 1. PROJECT OVERVIEW & HYDRAULIC DESIGN SUMMARY

This manual governs the complete field installation of the pressurized drip irrigation infrastructure serving 2.0 gross acres divided into two distinct agro-horticultural zones:

### Zone Breakdown

- Zone 1: Solar Borewell & Mainline Distribution Network

  - Water Source: 5.0 HP DC Solar Submersible Borewell at coordinate (X=590, Y=20)

  - Operating Pressure: 2.5 kg/cm² (35.5 PSI) dynamic delivery pressure

  - Discharge Rate: 18,000–22,000 Liters Per Hour (LPH) depending on solar irradiance

  - Mainline: 2" (63mm OD) PVC Class 2 (6 kg/cm² pressure rated) solvent-weld network

  - Total Mainline Run: 730 ft installed (80 ft North + 90 ft West to Tee A + 215 ft West to Tee B + 145 ft North Plot A + 240 ft North Plot B)

- Zone 2: Plot A (East Field) — PKM-1 Annual Drumstick

  - Net Planting Area: 1.0 Acre (390 ft East-West x 130 ft North-South)

  - Crop Spacing: High-density 3 ft plant-to-plant along trench x 5 ft between trenches

  - Total Population: 3,000 Moringa trees (leaf meal & pod production)

  - Riser Configuration: 4 Overhead Double-Tee Stations (A1–A4 spaced at 30 ft intervals)

  - Subline: 1.5" (50mm OD) Class 2 PVC (120 ft total run, 8 arms of 15 ft each)

  - Lateral Specification: 16x4x40 Inline Pipe (16mm LLDPE with integrated 4 LPH labyrinth drippers spaced every 40cm); 24 trenches x 390 ft = 2,926m installed

  - Take-off Connection: 50x16x2 Way Take-Off Collar (V-Connector) feeding East & West simultaneously

- Zone 3: Plot B (West Field) — Bhagya (KDM-01) Drumstick

  - Net Planting Area: 1.0 Acre (165 ft East-West x 260 ft North-South)

  - Crop Spacing: Commercial orchard 7 ft plant-to-plant x 8 ft between rows

  - Total Population: 750 Moringa trees (large fruit/export quality)

  - Riser Configuration: 4 Overhead Double-Tee Stations (B1–B4 spaced at 64 ft intervals)

  - Subline: 1.25" (40mm OD) Class 2 PVC (256 ft total run, 8 arms of 32 ft each)

  - Lateral Specification: Plain 16mm LLDPE pipe; 32 rows x 165 ft = 1,610m installed (Westward run only)

  - Emitter Specification: Tunning Dripper (P) (0–70 LPH adjustable turbulent flow) punched on-site every 7 ft (768 drippers total)

  - Take-off Connection: 40x16x1Way TF Collar pointing West

## 2. EXCAVATION & TRENCHING SPECIFICATIONS

Precise earthwork ensures pipeline longevity, prevents mechanical damage from inter-cultivation tractors/rotavators, and guarantees freeze/heat insulation.

| Pipeline Element | Nominal Size | Minimum Trench Depth | Minimum Trench Width | Bedding Material |
| --- | --- | --- | --- | --- |
| Zone 1 Mainline | 2" (63mm) PVC | 2'-6" (750 mm) | 1'-6" (450 mm) | 2" sifted sand or loose soil |
| Plot A Sublines | 1.5" (50mm) PVC | 2'-0" (600 mm) | 1'-2" (350 mm) | Fine rock-free soil bed |
| Plot B Sublines | 1.25" (40mm) PVC | 2'-0" (600 mm) | 1'-2" (350 mm) | Fine rock-free soil bed |
| Plot A Trenches | 24 Trenches | 0'-8" to 1'-0" (250 mm) | 1'-0" (300 mm) | Organic compost & soil mix |
| Plot B Rows | 32 Surface Rows | Surface / 2" furrow | N/A | Surface laid along row stake |

### Trenching Rules:

- Bottom Grading: Trenches must be free of sharp rocks, shale, roots, and clods larger than 10mm.

- Expansion Looping: Never lay PVC pipes dead straight under tension. Snake the pipe slightly inside the trench (1 ft lateral deflection per 100 ft) to accommodate thermal contraction during winter nights.

- Backfill Protocol: The first 6 inches of backfill must be hand-tamped sifted topsoil. Do NOT run heavy tractors over trenches until 24 hours after full backfilling.

## 3. HEAD UNIT & FILTRATION ASSEMBLY (ZONE 1)

Located at coordinate (590, 20) directly adjacent to the 5 HP Solar Array:

[Borewell Discharge Pipe]

│

▼

[2" Non-Return Valve (NRV)] ── Prevents water hammer back to pump impellers

│

▼

[2" PVC Gate Valve #1] ────── Primary flow isolation and throttling

│

▼

[2" Filter 'T' Type] ──────── 120-mesh disc/screen filter (Item #11)

│

▼

[Pressure Gauge Port] ─────── 0–6 kg/cm² glycerin-filled gauge

│

▼

[2" Borewell Tee (590, 20)] ── South outlet capped with 2" End Cap; North feeds Mainline

### Head Unit Installation Steps:

- Mount the 2" T-Type Filter vertically or horizontally with the drain/flushing port facing downward.

- Ensure flow arrow on the filter body points North toward the mainline.

- Apply 5–6 wraps of Teflon tape on all male threaded transitions before screwing into the filter housing. Do NOT use solvent cement on threaded filter collars.

- Provide a brick or concrete pedestal under the filter and gate valve to support hydraulic dynamic load.

## 4. 2" MAINLINE NETWORK LAYOUT & SOLVENT WELDING

### Master Alignment Coordinates:

- Station 1 (Borewell Tee): (590, 20) — South capped with 2" End Cap.

- Run 1: Northward run of 80'-0" from (590, 20) to (590, 100).

- Station 2 (Corner Tee): (590, 100) — East outlet capped with 2" End Cap.

- Run 2: Westward run of 90'-0" from (590, 100) to (500, 100) (Tee A).

- Run 3 (Inter-Plot Mainline): Westward run of 215'-0" from Tee A (500, 100) to Tee B (285, 100).

  - Runs under the West half of Plot A (190 ft) + across the 25 ft buffer roadway.

- Station 3 (Tee B): (285, 100) — West outlet capped with 2" End Cap.

### PVC Solvent Welding Best Practices:

- Cutting: Cut pipes squarely using a fine-tooth hacksaw blade or ratcheting PVC cutter. Remove all internal and external burrs with a deburring chamfer tool.

- Dry Fit: Check dry fit of pipe into fitting socket; it should enter 1/3 to 2/3 of the depth snugly.

- Cleaning: Wipe pipe ends and fitting interior with a dry cotton cloth.

- Solvent Application: Using the provided 3L heavy-duty PVC Solvent Cement, apply a uniform, generous layer around the pipe exterior and a thin, even layer inside the socket.

- Assembly: Push the pipe firmly home with a 1/4 turn twist to distribute the cement evenly. Hold rigidly for 30 seconds to prevent push-out.

- Cure Time: Allow minimum 2 hours initial set before moving pipes, and 24 hours full cure before pressure testing.

## 5. 4-FOOT OVERHEAD DOUBLE-TEE RISER MANIFOLD ASSEMBLY

The proprietary overhead double-tee manifold provides waist-height ergonomics (+4'-0" above finished grade), eliminating the need for operators to crawl into muddy pits to turn valves or monitor subline pressures.

### Assembly Sequence (Standard for all 8 Riser Stations):

- Buried Mainline Tee: In the mainline trench bottom (-2'-6"), solvent-weld a 2" PVC Equal Tee with the side outlet pointing vertically (+Z).

- Vertical Riser Pipe: Glue a 4'-0" cut length of 2" PVC pipe into the tee outlet, extending straight up through finished grade level to +4.0 ft.

- Overhead Double-Tee: Glue a 2" PVC Equal Tee at the top of the 4ft riser, oriented horizontally with branches facing North and South.

- Reducers:

  - Plot A Risers (A1–A4): Glue a 2" x 1.5" (50mm) PVC Bushing Reducer into both North and South tee outlets.

  - Plot B Risers (B1–B4): Glue a 2" x 1.25" (40mm) PVC Bushing Reducer into both North and South tee outlets.

- Ball Valves:

  - Plot A: Solvent-weld a 50mm (1.5") Plain Ball Valve (Item #13) onto each reducer stub.

  - Plot B: Solvent-weld a 40mm (1.25") Plain Ball Valve (Item #12) onto each reducer stub.

  - Ensure valve handle operates smoothly and points upwards.

- Overhead Elbow & Drop Pipe:

  - Glue a 90° PVC Elbow onto the valve discharge pointing directly downward.

  - Solvent-weld a 6'-0" cut length of drop pipe (1.5" for Plot A, 1.25" for Plot B) down into the subline trench (-2'-0" elevation).

- Trench Elbow & Subline Connection:

  - Glue a bottom 90° PVC Elbow at the trench bottom, turning horizontally into the subline trench.

  - Connect the horizontal buried subline pipe.

- Subline Flush Valve:

  - At the terminal end of every subline arm, install a 50mm Flush Valve (Plot A) or 40mm Flush Valve (Plot B).

  - Enclose the flush valve in an 8" PVC sleeve pipe with a removable cap flush with the ground for easy flushing access.

## 6. ZONE 2 (PLOT A) EXECUTION: PKM-1 MORINGA INLINE SYSTEM

### Spacing & Layout:

- Mainline Axis: Runs North from Tee A (500, 100) to (500, 245) (145 ft total, capped with 2" End Cap at (500, 245)).

- Riser Spacing: 4 Riser Stations spaced every 30'-0":

  - Riser A1: (500, 130) — Subline arms cover Y = 115' to 145'

  - Riser A2: (500, 160) — Subline arms cover Y = 145' to 175'

  - Riser A3: (500, 190) — Subline arms cover Y = 175' to 205'

  - Riser A4: (500, 220) — Subline arms cover Y = 205' to 235'

- Subline Arms: 8 arms total, each running 15'-0" (30 ft total per station; 120 ft total 1.5" subline).

- Trenches: 24 trenches spaced exactly 5'-0" apart (3 trenches per 15-ft arm x 8 arms = 24 trenches).

### Collar Drilling & Lateral Unrolling:

- Drilling: Using a 16mm spade bit or hole saw, drill into the top center of the buried 1.5" PVC subline at exact 5-foot intervals. Clean out all plastic shavings from inside the pipe.

- Collar Installation: Seat the rubber grommet firmly into the hole. Clamp the 50x16x2 Way Take of Coller (Item #9) around the 50mm pipe, securing the barbed stem through the grommet.

- Lateral Laying: Unroll the 16x4x40 Inline Pipe (Item #8) carefully along each trench bed. Do not drag the pipe over rough gravel to avoid scratching the LLDPE wall.

- Length: Cut each trench lateral to 390 ft total length (~195 ft West and ~195 ft East of the central subline).

- Connection: Push the East and West pipe ends firmly over the two barbed outlets of the 2-way collar. Secure with safety lock rings.

- End Capping: At the far East (X = 695) and far West (X = 305) ends, install 16mm Endcaps (Item #4) with figure-8 folds.

## 7. ZONE 3 (PLOT B) EXECUTION: BHAGYA MORINGA ONLINE SYSTEM

### Spacing & Layout:

- Mainline Axis: Runs North along the East boundary from Tee B (285, 100) to (285, 340) (240 ft total, capped with 2" End Cap at (285, 340)).

- Riser Spacing: 4 Riser Stations spaced every 64'-0":

  - Riser B1: (285, 124) — Subline arms cover Y = 92' to 156'

  - Riser B2: (285, 188) — Subline arms cover Y = 156' to 220'

  - Riser B3: (285, 252) — Subline arms cover Y = 220' to 284'

  - Riser B4: (285, 316) — Subline arms cover Y = 284' to 348'

- Subline Arms: 8 arms total, each running 32'-0" (64 ft total per station; 256 ft total 1.25" subline).

- Rows: 32 rows spaced exactly 8'-0" apart (4 rows per 32-ft arm x 8 arms = 32 rows).

### Collar & Dripper Installation:

- Collar Installation: Drill 16mm holes in the 1.25" PVC subline at 8-foot centers pointing West only. Install the 40x16x1Way TF Coller (Item #6) with rubber seal.

- Plain Pipe Laying: Unroll plain 16mm Pipe (Item #1) along each row for 165'-0" West (from X = 285 to X = 120). Connect to collar barb.

- Dripper Punching:

  - Measure 7'-0" from the plant base along the lateral.

  - Using the supplied Punching Tool (Item #3), punch a clean 3mm hole in the top quadrant of the lateral pipe.

  - Immediately press the barbed base of a Tunning Dripper (P) (Item #2) into the punched hole until it clicks audibly into place.

  - Repeat for 24 plants per row (24 x 32 = 768 drippers total).

- End Capping: Fold and secure the West end of all 32 rows with 16mm Endcap (Item #4) at X = 120.

## 8. SYSTEM FLUSHING & PRESSURE TESTING PROTOCOL

Flushing removes sand, PVC burrs, cement debris, and insects before water enters sensitive dripper labyrinths. Flushing must always proceed strictly from largest diameter to smallest diameter.

[STAGE A: Mainline Flush]

└── Remove 2" end caps at (500, 245) & (285, 340) -> Run pump at full flow for 5 minutes.

│

▼

[STAGE B: Subline Flush]

└── Close mainline caps; open all 16 Flush Valves (50mm & 40mm) -> Flush until water runs clear.

│

▼

[STAGE C: Lateral Flush]

└── Close flush valves; remove 16mm end caps from laterals -> Flush for 3 minutes per zone.

│

▼

[STAGE D: Pressure & Leak Test]

└── Cap all ends; pressurize system to 2.5 kg/cm² for 60 minutes -> Inspect all 120+ joints.

### Pressure Test Acceptance Criteria:

- Zero weeping or dripping at all solvent-welded joints, ball valve unions, and filter flanges.

- Pressure loss after pump shutoff must not exceed 0.2 kg/cm² in 15 minutes.

- All 3,000 inline drippers in Plot A and 768 online drippers in Plot B must show uniform wetting patterns with no dry skips or blowing emitters.

## 9. OPERATION & PREVENTATIVE MAINTENANCE

### Recommended Daily Irrigation Schedule:

- Summer (Peak Vegetative & Pod Stage): 35–45 Liters per plant daily.

  - Plot A (PKM-1): Run 2 Riser Stations simultaneously for 45 minutes; repeat for remaining 2 stations.

  - Plot B (Bhagya): Run 2 Riser Stations simultaneously for 40 minutes (drippers tuned to 35 LPH).

- Winter / Rainy Season: 15–20 Liters per plant every 2–3 days.

### Routine Maintenance Schedule:

- Daily: Open filter drain ball valve for 15 seconds while pump runs to purge trapped silt.

- Weekly: Unscrew filter housing, remove disc cartridge, and wash under high-pressure hose.

- Monthly: Open all 16 subline flush valves and 80 lateral endcaps for 2 minutes to flush accumulated silt.

- Seasonal (Every 6 Months): Acid treatment using 0.5% commercial Hydrochloric Acid (HCl) to dissolve calcium carbonate scaling and fertilizer salts in emitter labyrinths.